THE IMAGINATION UNIVERSITY PROGRAMME

RVfpga-SoC Lab 5

Tensorflow LITE

# Introduction :

In this Lab, we show how to build a Tensorflow LITE project (hello_world) for Zephyr that is a real-time operating system (RTOS), and then run on SweRVolf.

# Requirements :

To complete this lab, you will need to install the following:

- Verilator (v4.106) 			   (Refer to Installation Guide (Page No.13))

- FuseSoC 				   (Refer to Installation Guide (Page No.14))

- OpenOCD (RISC-V-specific version)	   (Refer to Installation Guide (Page No.14))

- Zephyr	Prerequisites		    	   (Refer to Installation Guide (Page No.15))

- Zephyr SDK (v0.12.4)			   (Refer to Installation Guide (Page No.15))

- PuTTY

IMPORTANT: Before starting RVfpga-SoC Labs, we highly recommend completing the RVfpga-SoC Installation Guide.

For example, if you have not already, install Xilinx’s Vivado and Verilator following the instructions in the RVfpga-SoC Installation Guide. Make sure that you have copied the RVfpga-SoC folder that you downloaded from Imagination’s University Programme to your machine.

# Tensorflow-Lite Overview:

TensorFlow Lite is a set of tools that enables on-device machine learning by helping developers run their models on mobile, embedded, and IoT devices.

Its key features are:

- Optimized for on-device machine learning, by addressing 5 key constraints: latency (there's no round-trip to a server), privacy (no personal data leaves the device), connectivity (internet connectivity is not required), size (reduced model and binary size), and power consumption (efficient inference and a lack of network connections).

- Multiple platform support, covering Android and iOS devices, embedded Linux, and microcontrollers.

- Diverse language support, which includes Java, Swift, Objective-C, C++, and Python.

- High performance, with hardware acceleration and model optimization.

- End-to-end examples, for common machine learning tasks such as image classification, object detection, pose estimation, question answering, text classification, etc. on multiple platforms.

# Tensorflow’s Hello World Example :

The Hello World example is designed to demonstrate the absolute basics of using TensorFlow Lite for Microcontrollers. We train and run a model that replicates a sine function, i.e, it takes a single number as its input and outputs the number's sine value. When deployed to the microcontroller, its predictions are used to either blink LEDs or control an animation.

The end-to-end workflow involves the following steps:

Train a model (in Python): A jupyter notebook to train, convert and optimize a model for on-device use.

Run inference (in C++ 11): An end-to-end unit test that runs inference on the model using the C++ library.

The following sections walk through the example's hello_world_test.cc, a unit test that demonstrates how to run inference using TensorFlow Lite for Microcontrollers. It loads the model and runs inference several times.

### 1. Include the library headers

To use the TensorFlow Lite for Microcontrollers library, we must include the following header files:

#include "tensorflow/lite/micro/all_ops_resolver.h" #include "tensorflow/lite/micro/micro_error_reporter.h" #include "tensorflow/lite/micro/micro_interpreter.h" #include "tensorflow/lite/schema/schema_generated.h" #include "tensorflow/lite/version.h"

- all_ops_resolver.h provides the operations used by the interpreter to run the model.

- micro_error_reporter.h outputs debug information.

- micro_interpreter.h contains code to load and run models.

- schema_generated.h contains the schema for the TensorFlow Lite FlatBuffer model file format.

- version.h provides versioning information for the TensorFlow Lite schema.

### 2. Include the model header

The TensorFlow Lite for Microcontrollers interpreter expects the model to be provided as a C++ array. The model is defined in model.h and model.cc files. The header is included with the following line:

#include "tensorflow/lite/micro/examples/hello_world/model.h"

### 3. Include the unit test framework header

In order to create a unit test, we include the TensorFlow Lite for Microcontrollers unit test framework by including the following line:

#include "tensorflow/lite/micro/testing/micro_test.h"

The test is defined using the following macros:

TF_LITE_MICRO_TESTS_BEGIN TF_LITE_MICRO_TEST(LoadModelAndPerformInference) { . // add code here . } TF_LITE_MICRO_TESTS_END

We now discuss the code included in the macro above.

### 4. Set up logging

To set up logging, a tflite::ErrorReporter pointer is created using a pointer to a tflite::MicroErrorReporter instance:

tflite::MicroErrorReporter micro_error_reporter; tflite::ErrorReporter* error_reporter = &micro_error_reporter;

This variable will be passed into the interpreter, which allows it to write logs. Since microcontrollers often have a variety of mechanisms for logging, the implementation of tflite::MicroErrorReporter is designed to be customized for your particular device.

### 5. Load a model

In the following code, the model is instantiated using data from a char array, g_model, which is declared in model.h. We then check the model to ensure its schema version is compatible with the version we are using:

const tflite::Model* model = ::tflite::GetModel(g_model); if (model->version() != TFLITE_SCHEMA_VERSION) { TF_LITE_REPORT_ERROR(error_reporter, "Model provided is schema version %d not equal " "to supported version %d.\n", model->version(), TFLITE_SCHEMA_VERSION); }

### 6. Instantiate operations resolver

An AllOpsResolver instance is declared. This will be used by the interpreter to access the operations that are used by the model:

tflite::AllOpsResolver resolver;

The AllOpsResolver loads all of the operations available in TensorFlow Lite for Microcontrollers, which uses a lot of memory. Since a given model will only use a subset of these operations, it's recommended that real-world applications load only the operations that are needed.

This is done using a different class, MicroMutableOpResolver. You can see how to use it in the Micro speech example's micro_speech_test.cc.

### 7. Allocate memory

We need to preallocate a certain amount of memory for input, output, and intermediate arrays. This is provided as a uint8_t array of size tensor_arena_size:

const int tensor_arena_size = 2 * 1024; uint8_t tensor_arena[tensor_arena_size];

The size required will depend on the model you are using and may need to be determined by experimentation.

### 8. Instantiate interpreter

We create a tflite::MicroInterpreter instance, passing in the variables created earlier:

tflite::MicroInterpreter interpreter(model, resolver, tensor_arena, tensor_arena_size, error_reporter);

### 9. Allocate tensors

We tell the interpreter to allocate memory from the tensor_arena for the model's tensors:

interpreter.AllocateTensors();

### 10. Validate input shape

The MicroInterpreter instance can provide us with a pointer to the model's input tensor by calling .input(0), where 0 represents the first (and only) input tensor:

// Obtain a pointer to the model's input tensor TfLiteTensor* input = interpreter.input(0);

We then inspect this tensor to confirm that its shape and type are what we are expecting:

// Make sure the input has the properties we expect TF_LITE_MICRO_EXPECT_NE(nullptr, input); // The property "dims" tells us the tensor's shape. It has one element for // each dimension. Our input is a 2D tensor containing 1 element, so "dims" // should have size 2. TF_LITE_MICRO_EXPECT_EQ(2, input->dims->size); // The value of each element gives the length of the corresponding tensor. // We should expect two single element tensors (one is contained within the // other). TF_LITE_MICRO_EXPECT_EQ(1, input->dims->data[0]); TF_LITE_MICRO_EXPECT_EQ(1, input->dims->data[1]); // The input is a 32 bit floating point value TF_LITE_MICRO_EXPECT_EQ(kTfLiteFloat32, input->type);

The enum value kTfLiteFloat32 is a reference to one of the TensorFlow Lite data types, and is defined in common.h.

### 11. Provide an input value

To provide input to the model, we set the contents of the input tensor, as follows:

input->data.f[0] = 0.;

In this case, we input a floating-point value representing 0.

### 12. Run the model

To run the model, we can call Invoke() on our tflite::MicroInterpreter instance:

TfLiteStatus invoke_status = interpreter.Invoke(); if (invoke_status != kTfLiteOk) { TF_LITE_REPORT_ERROR(error_reporter, "Invoke failed\n"); }

We can check the return value, a TfLiteStatus, to determine if the run was successful. The possible values of TfLiteStatus, defined in common.h, are kTfLiteOk and kTfLiteError.

The following code asserts that the value is kTfLiteOk, meaning inference was successfully run.

TF_LITE_MICRO_EXPECT_EQ(kTfLiteOk, invoke_status);

### 13. Obtain the output

The model's output tensor can be obtained by calling output(0) on the tflite::MicroInterpreter, where 0 represents the first (and only) output tensor.

In the example, the model's output is a single floating point value contained within a 2D tensor:

TfLiteTensor* output = interpreter.output(0); TF_LITE_MICRO_EXPECT_EQ(2, output->dims->size); TF_LITE_MICRO_EXPECT_EQ(1, input->dims->data[0]); TF_LITE_MICRO_EXPECT_EQ(1, input->dims->data[1]); TF_LITE_MICRO_EXPECT_EQ(kTfLiteFloat32, output->type);

We can read the value directly from the output tensor and assert that it is what we expect:

// Obtain the output value from the tensor float value = output->data.f[0]; // Check that the output value is within 0.05 of the expected value TF_LITE_MICRO_EXPECT_NEAR(0., value, 0.05);

### 14. Run inference again

The remainder of the code runs inference several more times. In each instance, we assign a value to the input tensor, invoke the interpreter, and read the result from the output tensor:

input->data.f[0] = 1.; interpreter.Invoke(); value = output->data.f[0]; TF_LITE_MICRO_EXPECT_NEAR(0.841, value, 0.05); input->data.f[0] = 3.; interpreter.Invoke(); value = output->data.f[0]; TF_LITE_MICRO_EXPECT_NEAR(0.141, value, 0.05); input->data.f[0] = 5.; interpreter.Invoke(); value = output->data.f[0]; TF_LITE_MICRO_EXPECT_NEAR(-0.959, value, 0.05);

### 15. Read the application code

Once you have walked through this unit test, you should be able to understand the example's application code, located in main_functions.cc. It follows a similar process, but generates an input value based on how many inferences have been run, and calls a device-specific function that displays the model's output to the user.

# Setting up The Environment For Tensorflow:

Open your Ubuntu terminal and complete the following steps :

Step 1. Navigate to the directory  “SweRVolf”. We have to set the following shell variables. To do that, we run the following:

$ export WORKSPACE=$(pwd)

$ export SWERVOLF_ROOT=$WORKSPACE/fusesoc_libraries/swervolf

$ export ZEPHYR_BASE=$WORKSPACE/zephyr

Figure 1. Set the shell variables

Step 2. Clone the Tensorflow GitHub repository.

$ git clone https://github.com/AlexeyYukhin/tensorflow

Figure 2. Tensorflow

Now navigate to the “tensorflow” directory.

$ cd tensorflow

Figure 3. Navigate to the “tensorflow” directory

Checkout the specific branch of the repository by the following command :

$ git checkout -b adc570a50410be7aba1c33522854f45fa0f349be

Figure 4. git checkout

Step 3. Navigate to the “zephyr” directory to install the required packages.

$ cd ../zephyr/

Figure 5. Navigate to the “zephyr” directory

Install the required packages listed in the “scripts“ or requirements.txt” file using the following command.

$ pip3 install -r scripts/requirements.txt

Figure 6. Installing required packages

Step 4. Create a virtual environment using the following command.

$ virtualenv venv-zephyr

Figure 7. Creating venv-zephyr

Step 5. Open a new terminal tab using “Ctrl + Shift + T”. Enter the following command in the new terminal tab to activate the virtual environment that we have created in the last step.

$ source venv-zephyr/bin/activate

Figure 8. Activating venv-zephyr

Step 6. Install the packages in the virtual environment.

$ sudo apt install git cmake ninja-build gperf ccache dfu-util device-tree-compiler wget python python3-pip python3-setuptools python3-tk python3-wheel xz-utils file make gcc gcc-multilib locales tar curl unzip xxd make autoconf g++ flex bison virtualenv

Figure 9. Installing packages in venv-zephyr

Install the “pyyaml” package using the following command:

$ pip install pyyaml

Figure 10. Installing pyyaml in venv-zephyr

also, install the “pyelftools” package using the following command:

$ pip install pyelftools

Figure 11. Installing pyelftools in venv-zephyr

Now we can close this terminal tab and return to our main terminal tab where we will be building the “hello_world” example.

# Building Hello World Example for Swervolf:

In this section, we will be building the “hello_world” example for Swervolf. We will be generating the “zephyr.bin” and “zephyr.elf” files for the “hello_world” example.

First, we will navigate to the tensorflow directory

$ cd ../tensorflow/

Figure 12. Navigating to the “tensorflow” directory

To build a runnable binary for a given project (such as an example application), we can use the following command, replacing <project_name> with the project we wish to build:

$ make -f tensorflow/lite/micro/tools/make/Makefile <project_name>_bin

As we are building binaries for the “hello_world” project so we can replace the <project_name> with “hello_world”.

$ make -f tensorflow/lite/micro/tools/make/Makefile hello_world_bin

We will target it for Swervolf using the parameter “TARGET”.

Now run the following command :

$ make -f tensorflow/lite/micro/tools/make/Makefile TARGET=zephyr_swervolf BUILD_TYPE=debug hello_world_bin

Figure 13. Building hello_world example

This will take a few minutes since it has to download some toolchains for the dependencies. Once it has finished, you should see some folders created inside a path like

tensorflow/lite/micro/tools/make/gen/zephyr_swervolf_x86_64_debug/hello_world/

These folders contain the generated project and source files.

Figure 14. Building hello_world example completed

The resulting binaries (zephyr.bin and zephyr.elf) will be generated in the following path:

tensorflow/lite/micro/tools/make/gen/zephyr_swervolf_x86_64_debug/hello_world

/build/zephyr

# Running Hello World Example on Verilator:

In this section, we will be converting the “zephyr.bin” file into a “.hex” file and then load it in as the initial ram file while running the simulator for swervolf.

Step 1. Navigate to the “hello_world” project directory. Enter the following command to enter that directory:

$ cd tensorflow/lite/micro/tools/make/gen/zephyr_swervolf_x86_64_debug/hello_world/

Figure 15. “hello_world” project path

Step 2. Convert the “.bin” file to the “.hex” file. To create the “.hex” file, run the following command from the hello_world directory :

$ python3 $SWERVOLF_ROOT/sw/makehex.py build/zephyr/zephyr.bin  > /home/{YourUsername}/RVfpgaSoC/Labs/LabProjects/SweRVolf/hello_world_tensorflow.hex

Figure 16. convert “.bin” to “.hex”

Step 3. Navigate back to the “WORKSPACE” directory

Figure 9. Installing packages in venv-zephyr

Step 4. Load the “.hex” file in the simulator :

$ fusesoc run --target=sim swervolf --ram_init_file=hello_world_tensorflow.hex

Figure 17. Loading “.hex” file in the simulator

We can see the output of the hello_world example (See Figure 18).

The program prints the “X” and “Y” coordinates of the sine function that the TensorFlow model is plotting.

Figure 18. “hello_world” output

# Running Hello World Example on the Nexys A7 Board:

In this section, we will be running the “hello_world” project on the board using OpenOCD.

Step 1. Connect the Nexys A7 board to your computer and then run the FPGA build command in the Workspace directory.

$ fusesoc run --target=nexys_a7 --run swervolf

Figure 19. Run the FPGA build

Step 2. Program the board with OpenOCD.

$ openocd -c "set BITFILE build/swervolf_0.7.3/nexys_a7-vivado/swervolf_0.7.3.bit" -f $SWERVOLF_ROOT/data/swervolf_nexys_program.cfg

Figure 20. Run OpenOCD

Step 3. Connect OpenOCD with SweRVolf.

$ openocd -f $SWERVOLF_ROOT/data/swervolf_nexys_debug.cfg

Figure 21. OpenOCD connected

Step 3. Open a new terminal using “Ctrl + Shift + t” & connect to the debug session through  OpenOCD using the following command:

$ telnet localhost 4444

Figure 22. telnet localhost 4444

OpenOCD supports loading ELF program files by running load_image /path/to/file.elf. Remember that the path is relative to the directory from where OpenOCD was launched.

> load_image tensorflow/tensorflow/lite/micro/tools/make/gen/zephyr_swervolf_x86_64_debug/hello_world/build/zephyr/zephyr.elf

Figure 23. loading the “.elf” file

After the program has been loaded, set the program counter to address zero using the following command:

> reg pc 0

Figure 24. Set program counter to zero

Now start the program using this command:

> resume

Figure 25. Start the program

Step 4. Open a new terminal using “Ctrl + Shift + t”. Open “PuTTY” using the command

$ putty

Figure 26. Open PuTTY

PuTTY is a free and open-source terminal emulator, serial console, and network file transfer application. We will be using it here as a serial console for our Nexys A7 board.

Step 5. Set the following configuration:

Select the connection type as “Serial”, then enter “/dev/ttyUSB1” as the serial line, and set the speed equal to “115200”.

Now click “Open” to start the serial console.

Figure 27. PuTTY configuration

In the serial console, we can see the output of the hello_world example (See Figure 28).

Again as we saw in the simulation section, the program prints the “X” and “Y” coordinates of the sine function that the TensorFlow model is plotting.

Figure 28. serial console